package org.firstinspires.ftc.teamcode.ColorFSMs;

import com.qualcomm.robotcore.eventloop.opmode.OpMode;

import com.qualcomm.robotcore.hardware.Servo;

import com.qualcomm.robotcore.util.ElapsedTime;

import org.firstinspires.ftc.teamcode.Backends.ColorSensorBackend;

public class ColorSensorFSM_PPG extends OpMode {

private ColorSensorBackend colorSensorBackend;

final double timer = 0.5;

final double timerforshoot = 0.75;

public enum Color_Sensor_PPG {

TRIGGER_PPG,

SHOOTPURPLE1_PPG,

BREAK1_PPG,

SHOOTPURPLE2_PPG,

BREAK2_PPG,

SHOOTGREEN_PPG,

BREAK3_PPG,

RESETFLICKER_PPG;

}

Servo Flicker1;

Servo Flicker2;

Servo Flicker3;

private int usedSensorPurple2 = 0;

private int usedSensorPurple1 = 0;

//PURPLE PURPLE GREEN

Color_Sensor_PPG color_sensor_ppg = Color_Sensor_PPG.TRIGGER_PPG;

ElapsedTime color_sensor_ppg_timer = new ElapsedTime();

private void flickSensor(int sensorId) {

switch (sensorId) {

case 1:

Flicker1.setPosition(1.0);

break;

case 2:

Flicker2.setPosition(1.0);

break;

case 3:

Flicker3.setPosition(1.0);

break;

}

}

@Override

public void init() {

Flicker1 = hardwareMap.get(Servo.class, "Flicker1");

Flicker2 = hardwareMap.get(Servo.class, "Flicker2");

Flicker3 = hardwareMap.get(Servo.class, "Flicker3");

color_sensor_ppg_timer.reset();

Flicker1.setPosition(0);

Flicker2.setPosition(0);

Flicker3.setPosition(0);

colorSensorBackend = new ColorSensorBackend(hardwareMap);

}

@Override

public void loop() {

ColorSensorBackend.SensorResult[] sensors = colorSensorBackend.getDetectedColor(telemetry);

if (gamepad1.b) {

Flicker1.setPosition(1);

color_sensor_ppg_timer.reset();

color_sensor_ppg = Color_Sensor_PPG.TRIGGER_PPG;

//Can integrate the turret override on press of button as well in main

}

if (gamepad1.x) {

Flicker2.setPosition(1);

color_sensor_ppg_timer.reset();

color_sensor_ppg = Color_Sensor_PPG.TRIGGER_PPG;

}

if (gamepad1.y) {

Flicker3.setPosition(1);

color_sensor_ppg_timer.reset();

color_sensor_ppg = Color_Sensor_PPG.TRIGGER_PPG;

}

switch (color_sensor_ppg) {

case TRIGGER_PPG:

if (gamepad1.a) {

color_sensor_ppg_timer.reset();

color_sensor_ppg = Color_Sensor_PPG.SHOOTPURPLE1_PPG;

} break;

case SHOOTPURPLE1_PPG:

for (ColorSensorBackend.SensorResult sensor : sensors) {

if (sensor.color == ColorSensorBackend.detectedColor.PURPLE_DIRECT || sensor.color == ColorSensorBackend.detectedColor.PURPLE_INDIRECT) {

int sensorId = sensor.sensorId;

usedSensorPurple1 = sensorId;

flickSensor(sensorId);

color_sensor_ppg_timer.reset();

color_sensor_ppg = Color_Sensor_PPG.BREAK1_PPG;

break;

}

} break;

case BREAK1_PPG:

if (color_sensor_ppg_timer.seconds() >= timerforshoot) {

color_sensor_ppg_timer.reset();

color_sensor_ppg = Color_Sensor_PPG.SHOOTPURPLE2_PPG;

} break;

case SHOOTPURPLE2_PPG:

for (ColorSensorBackend.SensorResult sensor : sensors) {

if (sensor.color == ColorSensorBackend.detectedColor.PURPLE_DIRECT || sensor.color == ColorSensorBackend.detectedColor.PURPLE_INDIRECT &&

sensor.sensorId != usedSensorPurple1) {

int sensorId = sensor.sensorId;

usedSensorPurple2 = sensorId;

flickSensor(sensorId);

color_sensor_ppg_timer.reset();

color_sensor_ppg = Color_Sensor_PPG.BREAK2_PPG;

break;

}

} break;

case BREAK2_PPG:

if (color_sensor_ppg_timer.seconds() >= timerforshoot) {

color_sensor_ppg_timer.reset();

color_sensor_ppg = Color_Sensor_PPG.SHOOTGREEN_PPG;

}

break;

case SHOOTGREEN_PPG:

for (ColorSensorBackend.SensorResult sensor : sensors) {

if (sensor.color == ColorSensorBackend.detectedColor.GREEN_DIRECT || sensor.color == ColorSensorBackend.detectedColor.GREEN_INDIRECT &&

sensor.sensorId != usedSensorPurple1 &&

sensor.sensorId != usedSensorPurple2) {

int sensorId = sensor.sensorId;

flickSensor(sensorId);

color_sensor_ppg_timer.reset();

color_sensor_ppg = Color_Sensor_PPG.BREAK3_PPG;

break;

}

}

break;

case BREAK3_PPG:

if (color_sensor_ppg_timer.seconds() >= timerforshoot) {

color_sensor_ppg_timer.reset();

color_sensor_ppg = Color_Sensor_PPG.RESETFLICKER_PPG;

} break;

case RESETFLICKER_PPG:

if (color_sensor_ppg_timer.seconds() >= timer){

Flicker1.setPosition(0);

Flicker2.setPosition(0);

Flicker3.setPosition(0);

color_sensor_ppg_timer.reset();

color_sensor_ppg = Color_Sensor_PPG.TRIGGER_PPG;

} break;

}

}

}