package org.firstinspires.ftc.teamcode;

import com.qualcomm.robotcore.eventloop.opmode.OpMode;

import com.qualcomm.robotcore.eventloop.opmode.TeleOp;

import com.qualcomm.robotcore.hardware.ColorSensor;

import com.qualcomm.robotcore.hardware.HardwareMap;

import com.qualcomm.robotcore.hardware.NormalizedColorSensor;

import com.qualcomm.robotcore.hardware.NormalizedRGBA;

import com.qualcomm.robotcore.hardware.Servo;

import com.qualcomm.robotcore.util.ElapsedTime;

import org.firstinspires.ftc.teamcode.Components.ColorSensorBackend;

import org.firstinspires.ftc.robotcore.external.Telemetry;

@TeleOp (name="ColorSensorCode")

public class ColorSensorFSM extends OpMode {

private ColorSensorBackend colorSensorBackend;

final double timer = 0.5;

final double timerforshoot = 0.75;

public enum Color_Sensor_GPP {

TRIGGER,

SHOOTGREEN,

BREAK1,

SHOOTPURPLE1,

BREAK2,

SHOOTPURPLE2,

RESETFLICKERS

}

Servo Flicker1;

Servo Flicker2;

Servo Flicker3;

private int usedSensorGreen = 0;

private int usedSensorPurple1 = 0;

Color_Sensor_GPP color_sensor_gpp = Color_Sensor_GPP.TRIGGER;

ElapsedTime color_sensor_gpp_timer = new ElapsedTime();

private void flickSensor(int sensorId) {

switch (sensorId) {

case 1:

Flicker1.setPosition(1.0);

break;

case 2:

Flicker2.setPosition(1.0);

break;

case 3:

Flicker3.setPosition(1.0);

break;

}

}

@Override

public void init() {

Flicker1 = hardwareMap.get(Servo.class, "Flicker1");

Flicker2 = hardwareMap.get(Servo.class, "Flicker2");

Flicker3 = hardwareMap.get(Servo.class, "Flicker3");

color_sensor_gpp_timer.reset();

Flicker1.setPosition(0);

Flicker2.setPosition(0);

Flicker3.setPosition(0);

colorSensorBackend = new ColorSensorBackend(hardwareMap);

}

@Override

public void loop() {

ColorSensorBackend.SensorResult[] sensors = colorSensorBackend.getDetectedColor(telemetry);

switch (color_sensor_gpp) {

case TRIGGER:

if (gamepad1.a){

color_sensor_gpp_timer.reset();

color_sensor_gpp = Color_Sensor_GPP.SHOOTGREEN;

} break;

case SHOOTGREEN:

for (ColorSensorBackend.SensorResult sensor : sensors) {

if (sensor.color == ColorSensorBackend.detectedColor.GREEN_DIRECT || sensor.color == ColorSensorBackend.detectedColor.GREEN_INDIRECT) {

int sensorId = sensor.sensorId;

flickSensor(sensorId);

usedSensorGreen = sensor.sensorId;

color_sensor_gpp_timer.reset();

color_sensor_gpp = Color_Sensor_GPP.BREAK1;

break;

}

} break;

case BREAK1:

if (color_sensor_gpp_timer.seconds() >= timerforshoot){

color_sensor_gpp_timer.reset();

color_sensor_gpp = Color_Sensor_GPP.SHOOTPURPLE1;

} break;

case SHOOTPURPLE1:

for (ColorSensorBackend.SensorResult sensor : sensors) {

if (sensor.color == ColorSensorBackend.detectedColor.PURPLE_DIRECT || sensor.color == ColorSensorBackend.detectedColor.PURPLE_INDIRECT &&

sensor.sensorId != usedSensorGreen) {

int sensorId = sensor.sensorId;

usedSensorPurple1 = sensorId;

flickSensor(sensorId);

color_sensor_gpp_timer.reset();

color_sensor_gpp = Color_Sensor_GPP.BREAK2;

break;

}

} break;

case BREAK2:

if (color_sensor_gpp_timer.seconds() >= timerforshoot) {

color_sensor_gpp_timer.reset();

color_sensor_gpp = Color_Sensor_GPP.SHOOTPURPLE2;

} break;

case SHOOTPURPLE2:

for (ColorSensorBackend.SensorResult sensor : sensors) {

if (sensor.color == ColorSensorBackend.detectedColor.PURPLE_DIRECT || sensor.color == ColorSensorBackend.detectedColor.PURPLE_INDIRECT &&

sensor.sensorId != usedSensorGreen &&

sensor.sensorId != usedSensorPurple1) {

int sensorId = sensor.sensorId;

flickSensor(sensorId);

color_sensor_gpp_timer.reset();

color_sensor_gpp = Color_Sensor_GPP.RESETFLICKERS;

break;

}

} break;

case RESETFLICKERS:

if (color_sensor_gpp_timer.seconds() >= timer){

Flicker1.setPosition(0);

Flicker2.setPosition(0);

Flicker3.setPosition(0);

color_sensor_gpp_timer.reset();

color_sensor_gpp = Color_Sensor_GPP.TRIGGER;

break;

} break;

}

if (gamepad1.b){

Flicker1.setPosition(1);

color_sensor_gpp_timer.reset();

color_sensor_gpp = Color_Sensor_GPP.TRIGGER;

//Can integrate the turret override on press of button as well in main

}

if (gamepad1.x){

Flicker2.setPosition(1);

color_sensor_gpp_timer.reset();

color_sensor_gpp = Color_Sensor_GPP.TRIGGER;

}

if (gamepad1.y){

Flicker3.setPosition(1);

color_sensor_gpp_timer.reset();

color_sensor_gpp = Color_Sensor_GPP.TRIGGER;

}

}

}