package org.firstinspires.ftc.teamcode.CameraLogitech;

import com.qualcomm.robotcore.eventloop.opmode.LinearOpMode;

import com.qualcomm.robotcore.eventloop.opmode.TeleOp;

import com.qualcomm.robotcore.hardware.DcMotor;

import com.qualcomm.robotcore.util.Range;

import org.firstinspires.ftc.robotcore.external.hardware.camera.BuiltinCameraDirection;

import org.firstinspires.ftc.robotcore.external.hardware.camera.WebcamName;

import org.firstinspires.ftc.robotcore.external.hardware.camera.controls.ExposureControl;

import org.firstinspires.ftc.robotcore.external.hardware.camera.controls.GainControl;

import org.firstinspires.ftc.vision.VisionPortal;

import org.firstinspires.ftc.vision.apriltag.AprilTagDetection;

import org.firstinspires.ftc.vision.apriltag.AprilTagGameDatabase;

import org.firstinspires.ftc.vision.apriltag.AprilTagLibrary;

import org.firstinspires.ftc.vision.apriltag.AprilTagProcessor;

import java.util.List;

import java.util.concurrent.TimeUnit;

@TeleOp(name="Omni Drive To AprilTag", group = "Concept")

public class TurretTracking extends LinearOpMode {

// Adjust these numbers to suit your robot.

//  Set the GAIN constants to control the relationship between the measured position error, and how much power is

//  applied to the drive motors to correct the error.

//  Drive = Error * Gain    Make these values smaller for smoother control, or larger for a more aggressive response.

final double TURN_GAIN   =  0.01  ;   //  Turn Control "Gain".  e.g. Ramp up to 25% power at a 25 degree error. (0.25 / 25.0)

final double MAX_AUTO_TURN  = 0.3;   //  Clip the turn speed to this max value (adjust for your robot)

//TURRET INITIALIZATION!

private DcMotor TurretBoy = null;

//TURRET INITIALIZATION

private static final boolean USE_WEBCAM = true;  // Set true to use a webcam, or false for a phone camera

private static final int DESIRED_TAG_ID = -1;     // Choose the tag you want to approach or set to -1 for ANY tag.

private VisionPortal visionPortal;               // Used to manage the video source.

private AprilTagProcessor aprilTag;              // Used for managing the AprilTag detection process.

private AprilTagDetection desiredTag = null;     // Used to hold the data for a detected AprilTag

@Override public void runOpMode()

{

boolean targetFound     = false;    // Set to true when an AprilTag target is detected

double  turn            = 0;        // Desired turning power/speed (-1 to +1)

// Initialize the Apriltag Detection process

initAprilTag();

TurretBoy = hardwareMap.get(DcMotor.class,"TurretBoy");

TurretBoy.setMode(DcMotor.RunMode.RUN_WITHOUT_ENCODER);

TurretBoy.setZeroPowerBehavior(DcMotor.ZeroPowerBehavior.BRAKE);

if (USE_WEBCAM)

setManualExposure(6, 250);  // Use low exposure time to reduce motion blur

// Wait for driver to press start

telemetry.addData("Camera preview on/off", "3 dots, Camera Stream");

telemetry.addData(">", "Touch START to start OpMode");

telemetry.update();

waitForStart();

while (opModeIsActive())

{

targetFound = false;

desiredTag  = null;

// Step through the list of detected tags and look for a matching tag

List<AprilTagDetection> currentDetections = aprilTag.getDetections();

for (AprilTagDetection detection : currentDetections) {

// Look to see if we have size info on this tag.

if (detection.id >= 20 && detection.id <= 24) {

//  Check to see if we want to track towards this tag.

if ((DESIRED_TAG_ID < 0) || (detection.id == DESIRED_TAG_ID)) {

// Yes, we want to use this tag.

targetFound = true;

desiredTag = detection;

break;  // don't look any further.

} else {

// This tag is in the library, but we do not want to track it right now.

telemetry.addData("Skipping", "Tag ID %d is not desired", detection.id);

}

} else {

// This tag is NOT in the library, so we don't have enough information to track to it.

telemetry.addData("Unknown", "Tag ID %d is not in TagLibrary", detection.id);

}

}

// Tell the driver what we see, and what to do.

if (targetFound) {

telemetry.addData(">>>", "Tracking ID %d", desiredTag.id);

// Safe metadata

String tagName = (desiredTag.metadata != null) ? desiredTag.metadata.name : "UNKNOWN";

// Safe pose - CHECK before using

if (desiredTag.ftcPose != null) {

telemetry.addData("Tag", "%s (%.1fin, %.1f°)", tagName, desiredTag.ftcPose.range, desiredTag.ftcPose.bearing);

double headingError = desiredTag.ftcPose.bearing;

turn = Range.clip(headingError * TURN_GAIN, -MAX_AUTO_TURN, MAX_AUTO_TURN);

} else {

telemetry.addData("Tag", "%s (NO POSE)", tagName);

turn = 0;

}

} else {

turn = 0;

telemetry.addData("Status", "No 20-24 tags detected");

}

telemetry.addData("Turret Power", "%.2f", turn);

telemetry.update();

// Apply desired axes motions to the Turret.

MoveTurret(turn);

}

}

public void MoveTurret(double yaw) {

// Turret power

TurretBoy.setPower(yaw);

}

/**

* Initialize the AprilTag processor.

*/

private void initAprilTag() {

// Create the AprilTag processor by using a builder.

AprilTagLibrary lib = AprilTagGameDatabase.getCurrentGameTagLibrary();

aprilTag = new AprilTagProcessor.Builder()

.setTagLibrary(lib)

.build();

// Adjust Image Decimation to trade-off detection-range for detection-rate.

// e.g. Some typical detection data using a Logitech C920 WebCam

// Decimation = 1 ..  Detect 2" Tag from 10 feet away at 10 Frames per second

// Decimation = 2 ..  Detect 2" Tag from 6  feet away at 22 Frames per second

// Decimation = 3 ..  Detect 2" Tag from 4  feet away at 30 Frames Per Second

// Decimation = 3 ..  Detect 5" Tag from 10 feet away at 30 Frames Per Second

// Note: Decimation can be changed on-the-fly to adapt during a match.

aprilTag.setDecimation(2);

// Create the vision portal by using a builder.

if (USE_WEBCAM) {

visionPortal = new VisionPortal.Builder()

.setCamera(hardwareMap.get(WebcamName.class, "Webcam 1"))

.addProcessor(aprilTag)

.build();

} else {

visionPortal = new VisionPortal.Builder()

.setCamera(BuiltinCameraDirection.BACK)

.addProcessor(aprilTag)

.build();

}

}

/*

Manually set the camera gain and exposure.

This can only be called AFTER calling initAprilTag(), and only works for Webcams;

*/

private void    setManualExposure(int exposureMS, int gain) {

// Wait for the camera to be open, then use the controls

if (visionPortal == null) {

return;

}

// Make sure camera is streaming before we try to set the exposure controls

if (visionPortal.getCameraState() != VisionPortal.CameraState.STREAMING) {

telemetry.addData("Camera", "Waiting");

telemetry.update();

while (!isStopRequested() && (visionPortal.getCameraState() != VisionPortal.CameraState.STREAMING)) {

sleep(20);

}

telemetry.addData("Camera", "Ready");

telemetry.update();

}

// Set camera controls unless we are stopping.

if (!isStopRequested())

{

ExposureControl exposureControl = visionPortal.getCameraControl(ExposureControl.class);

if (exposureControl.getMode() != ExposureControl.Mode.Manual) {

exposureControl.setMode(ExposureControl.Mode.Manual);

sleep(50);

}

exposureControl.setExposure((long)exposureMS, TimeUnit.MILLISECONDS);

sleep(20);

GainControl gainControl = visionPortal.getCameraControl(GainControl.class);

gainControl.setGain(gain);

sleep(20);

}

}

}